# LAB02

Q1

data segment

n1 db 12H

n2 db ?

data ends

code segment

assume cs:code, ds:data

MOV AX, data

MOV ds, AX

LEA SI, n1

MOV BX, [SI]

INT 03

code ends

End

data segment

n1 db 12H

n2 db ?

n3 db ?

m1 dw 1273H

m2 dw ?

o1 dd 12341234H

o2 dd ?

data ends

code segment

assume cs:code, ds:data

MOV AX, data

MOV ds, AX

LEA SI, n1

MOV BL, [SI]

MOV n2, BL

MOV BL, ds:[0001h]

MOV n3, BL

MOV AX, m1

MOV m2, AX

MOV BX, word ptr o1

MOV word ptr o2, BX

MOV BX, word ptr 2[o1]

MOV word ptr 2[o2], BX

INT 03

code ends

end

Q2

data segment

n1 dw 1225H

n2 dw 3212H

n3 dw ?

data ends

code segment

assume cs:code, ds:data

MOV AX, data

MOV ds, AX

MOV AX, n1

ADD AX, n2

MOV n3, AX

INT 03

code ends

end

Q3

data segment

n1 dd 11111111H

n2 dd 22222222H

ans dd ?

data ends

code segment

assume cs:code, ds:data

MOV AX, data

MOV ds, AX

MOV AX, word ptr n1

ADD AX, word ptr n2

MOV BX, word ptr 2[n1]

ADC BX, word ptr 2[n2]

MOV word ptr ans, AX

MOV word ptr ans + 2, BX

INT 03

code ends

end

# LAB03

1. Program to multiply two unsigned 16-bit numbers.

data segment

num1 dw 0005H

num2 dw 0002H

data ends

code segment

ASSUME ds:data,cs:code

MOV AX, data

MOV ds, AX

MOV AX, num1

MOV BX, num2

MUL BX

INT 03H

code ends

end

2. Program to multiply signed 16-bit numbers.

data segment

num1 dw 0FFFEH

num2 dw 0FFFDH

data ends

code segment

ASSUME ds:data,cs:code

MOV AX, data

MOV ds, AX

MOV AX, num1

MOV BX, num2

IMUL BX

INT 03H

code ends

end

3. Program to divide a 16-bit unsigned/signed number by 16-bit number. The numbers are stored into the data segment.

data segment

num1 dw 0004H

num2 dw 0002H

num3 dw -0006

num4 dw -0002

data ends

code segment

ASSUME ds:data,cs:code

MOV AX, data

MOV ds, AX

MOV AX, num1

MOV BX, num2

DIV BX

MOV AX, num3

MOV BX, num4

CWD

IDIV BX

INT 03H

code ends

end

4. Find the Checksum and verify validity of a 32 bit data.

data segment

num1 dd 22221555H

num2 dd 23221555H

checksum db 00H

data ends

code segment

ASSUME ds:data,cs:code

MOV AX, data

MOV ds, AX

MOV AX, word ptr num1

MOV BX, word ptr num2

XOR AX, BX

JZ skip1

INC checksum

INT 03H

skip1:  MOV AX, word ptr 2[num1]

MOV BX, word ptr 2[num2]

XOR AX, BX

JZ end1

INC checksum

end1:   INT 03H

code ends

5. Program to copy an array of bytes/words from the variable “SOURCE” to

variable “DEST” which are defined in data segment.

data segment

source db "mfplab"

destination db ?

data ends

code segment

ASSUME ds:data,cs:code

MOV AX, data

MOV ds, AX

MOV SI, offset source

MOV BX, offset destination

MOV CX, 0006H

loop2:  MOV DL, [SI]

MOV [BX], DL

INC SI

INC BX

DEC CX

JNZ loop2

INT 03H

code ends

end

6. To sum an array of numbers stored in the data segment.

# LAB05

q. 1

data segment

number1 DQ 0000001212121212H

number2 DQ 0000002121212121H

answer DQ ?

data ends

code segment

assume ds:data, cs:code, es:code

MOV AX, data

MOV ds, AX

LEA SI, number1

LEA DI, answer

MOV CX, 3

loop1:    MOV AX, word ptr [SI]

MOV BX, word ptr [SI + 8H]

ADD AX, BX

MOV word ptr [DI], AX

ADD SI, 2

ADD DI, 2

LOOP loop1

INT 03H

code ends

end

q2

data segment

stringinput db 20,19 dup(0)

data ends

code segment

assume ds:data, cs:code, es:code

MOV AX, data

MOV ds, AX

MOV es, AX

LEA DX, stringinput

MOV ah, 10

INT 21H

mov cx, 0

MOV CL, byte ptr stringinput

LEA DI, stringinput

ADD DI, 2

MOV ah, 1

int 21h

CLD

repne scasb

JNE not_found

MOV BX, DI

not_found:

INT 03H

code ends

end

q.3

data segment

arr1 DW 1212H, 2323H, 3434H, 4545H

arr2 DW 4 dup(0)

data ends

code segment

assume ds:data, cs:code, es:code

MOV AX, data

MOV ds, AX

LEA SI, arr1

LEA DI, arr2

MOV CX, 4

loop1:    MOV AX, word ptr [SI]

MOV word ptr [DI], AX

ADD SI, 2

ADD DI, 2

LOOP loop1

INT 03H

code ends

end

ends

end

# LAB06

ASSIGNMENT 6

1. Write an assembly language to count the number of occurrence of a substring in a given string.

data segment

datastring      db      "HelloHelloHello"

lendata         db      ($ - datastring)

searchstring    db      "Hello"

lensearch       db      ($-searchstring)

data ends

code segment

assume ds:data, cs:code, es:data

MOV AX, data

MOV ds, AX

MOV es, AX

MOV BH, 0

MOV BL, byte ptr lendata

MOV DH, 0

MOV DL, byte ptr lensearch

SUB BX, DX

ADD BX, 1

LEA SI, datastring

LEA DI, searchstring

LEA SP, datastring

MOV AX, 0

outer:  MOV CX, DX

cld

rep cmpsb

JNZ nsame

INC AX

nsame:  INC SP

MOV SI, SP

LEA DI, datastring

DEC BX

JNZ outer

INT 03H

code ends

2. In a string “Good Morning”, write a program to replace the substring “Morning” with “Evening”

and display “Good Evening”.

data segment

parentstring    db      "Good Morning$"

lenparent       db      ($ - parentstring)

repstring       db      "Evening$"

lenrep          db      ($-repstring)

data ends

code segment

assume ds:data, cs:code, es:data

MOV AX, data

MOV ds, AX

MOV es, AX

LEA DI, parentstring

LEA SI, repstring

MOV CH, 0

MOV CL, byte ptr lenrep

ADD DI, 5

CLD

rep movsb

MOV DX, offset parentstring

MOV AH, 9

int 21H

INT 03H

code ends

end

3. Write a program to concatenate two strings STR1 and STR2 and store the result in the string

STR3. Try to input the string through the keyboard.

data segment

str1    db      10,9 dup(0)

str2    db      10,9 dup(0)

str3    db      20 dup(0)

data ends

code segment

assume ds:data, cs:code, es:data

MOV AX, data

MOV ds, AX

MOV es, AX

LEA DX, str1

MOV AH, 10

int 21H

LEA DX, str2

MOV AH, 10

int 21H

LEA SI, 2[str1]

LEA DI, str3

MOV CH, 0

MOV CL, byte ptr 1[str1]

rep movsb

LEA SI, 2[str2]

MOV CL, byte ptr 1[str2]

rep movsb

INT 03H

code ends

end

4. Program to enter two strings, Find the characters that match in both the strings, store these

Characters

data segment

str1	db	10,9 dup(0)

str2	db	10,9 dup(0)

samech	db	10 dup(0)

data ends

code segment

assume ds:data, cs:code, es:data

MOV AX, data

MOV ds, AX

MOV es, AX

LEA DX, str1

MOV AH, 10

int 21H

LEA DX, str2

MOV AH, 10

int 21H

LEA SI, 2[str1]

LEA DI, 2[str2]

MOV CH, 0

MOV BX, 0

MOV DL, byte ptr 1[str1]

outer:

MOV AL, byte ptr [SI]

repne scasb

JNE nfound

MOV samech[BX], AL

INC BX

nfound:

INC SI

MOV CL, byte ptr 1[str2]

LEA DI, 2[str2]

DEC DL

JNZ outer

INT 03H

code ends

end

# LAB07

1,Write the programs to verify the instructions AAA, AAS, AAM, AAD, DAA and DAS

Instructions.

code segment

;	MOV AX, 3135H

;	MOV BX, 3136H

;	ADD AX, BX

;	AAA

;	MOV AL, 09H

;	MUL BL

;	AAM

;	MOV AX, 39H

;	MOV AX, 24H

;	MOV BX, 19H

;	ADD AX, BX

;	DAA

;	SUB AX, BX

;	DAS

;	MOV AX, 0108H

;	MOV BX, 0009H

;	MUL BX

;	AAM

;	AAD

;	DIV BL

INT 03H

code ends

end

2. Implement a calculator for single digit numbers, which takes the input through the keyboard

and display the result on the screen.

data segment

op1	db	?

op2	db	?

opr	db	?

res 	dw	?

dol	db	"$"

nline	db	13, 10, "$"

data ends

code segment

ASSUME cs:code, ds:data

MOV AX, data

MOV ds, AX

loop1:

MOV AH, 01H

INT 21H

MOV op1, AL

INT 21H

MOV opr, AL

INT 21H

MOV op2, AL

AND op1, 0FH

AND op2, 0FH

MOV AH, 0

MOV AL, op1

CMP opr, '+'

JNE next1

ADD AL, op2

AAA

JMP exit

next1:

CMP opr, '-'

JNE next2

SUB AL, op2

AAS

JMP exit

next2:

CMP opr, '*'

JNE next3

MUL op2

AAM

JMP exit

next3:

CMP opr, '/'

JNE exit

MOV BL, op2

DIV BL

exit:

OR AX, 3030H

MOV byte ptr 1[res], AL

MOV byte ptr res, AH

MOV AH, 9

LEA DX, nline

INT 21H

MOV DX, offset res

INT 21H

LEA DX, nline

INT 21H

JMP loop1

INT 03H

code ends

end

data segment

op1	db	?

op2	db	?

opr	db	?

res 	dw	?

dol	db	"$"

nline	db	13, 10, "$"

data ends

code segment

ASSUME cs:code, ds:data

MOV AX, data

MOV ds, AX

loop1:

MOV AH, 01H

INT 21H

MOV op1, AL

INT 21H

MOV opr, AL

INT 21H

MOV op2, AL

AND op1, 0FH

AND op2, 0FH

MOV AH, 0

MOV AL, op1

CMP opr, '+'

JNE loop1

ADD AL, op2

CMP AL, 9

JC do

LAHF

AND AH, 8

JNE skip

ADD AL, 6

ADD AH, 1

skip:

OR AX, 3030H

MOV byte ptr 1[res], AL

MOV byte ptr res, AH

MOV AH, 9

LEA DX, nline

INT 21H

MOV DX, offset res

INT 21H

LEA DX, nline

INT 21H

JMP loop1

INT 03H

code ends

end

# LAB08

i.Write an assembly language program to convert the code of a 2 digit BCD number to hexadecimal and

Viceversa.

data SEGMENT

num_bcd DB 25H

num_hex DB 1AH

ans_hex DB 0

ans_bcd DB 0

data ends

code SEGMENT

ASSUME ds:data, cs:code

MOV AX, data

MOV ds, AX

MOV CL, 4

MOV AH, 0

;for bcd to hex

MOV AL, num_bcd

AND AL, 0FH

ADD ans_hex, AL

MOV AL, num_bcd

SHR AX, CL

MOV CH, 10

MUL CH

ADD ans_hex, AL

;for hex to bcd

MOV CL, num_hex

MOV CH, 0

MOV AL, 0

MOV AH, 1

addanddaa:

ADD AL, AH

DAA

LOOP addanddaa

MOV ans_bcd, AL

INT 03H

code ends

end

ii. Write an assembly language program to convert a number entered through keyboard to 7 segment

Display.

data segment

table   db  3fh, 06h, 5bh, 4fh, 66h, 6dh, 7dh, 07h, 7fh, 6fh

data ends

code segment

ASSUME cs:code, ds:data

MOV AX, data

MOV ds, AX

MOV AH, 1

INT 21H

AND AL, 0FH

LEA BX, table

XLAT

INT 03H

code ends

end

iii. Write an assembly language program to sort an array of words.

data segment

list1 dw 11, 6, 10, 25, 27, 18, 2, 35, 72, 1, 26

data ends

code segment

ASSUME cs:code, ds:data

MOV AX, data

MOV ds, AX

LEA SI, list1

MOV CX, word ptr [SI]

ADD SI, 2

DEC CX

MOV CH, CL

main_loop:

MOV AX, word ptr [SI]

CMP AX, word ptr 2[SI]

JC nswap

MOV BX, AX

MOV AX, word ptr 2[SI]

MOV word ptr [SI], AX

MOV word ptr 2[SI], BX

nswap:

ADD SI, 2

DEC CL

JNZ main_loop

LEA SI, list1

ADD SI, 2

DEC CH

MOV CL, CH

JNZ main_loop

INT 03H

code ends

end

# LAB09

Write a program to create a directory and in that create a file and write 10 bytes of data into the file.Create one more file and make a copy of the first file.(i.e Read from the first file and write into the second file.).

data segment

dir_name    db  "myDir", 0

file_name1  db  "myDir\newFile", 0

file_data1  db  "hello in file", 0

data_len    db  ($ - file_data1)

file_hd1    dw  ?

file_name2  db  "myDir\cpFile",0

file_hd2    dw  ?

temp_data   db  30 dup(0)

temp_len    dw  ?

data ends

code segment

assume ds:data, cs:code, es:data

MOV AX, data

MOV ds, AX

MOV es, AX

;creating a dir

LEA DX, dir_name

MOV AH, 39H

INT 21H

;creating a file

LEA DX, file_name1

MOV CX, 0

MOV AH, 3CH

INT 21H

MOV file_hd1, AX

;adding data in file

MOV BX, AX

MOV CX, 0

MOV CL, data_len

LEA DX, file_data1

MOV AH, 40H

INT 21H

;closing one file to save

MOV BX, file_hd1

MOV AH, 3EH

INT 21h

;opening file 1

MOV AL, 2

MOV AH, 3DH

LEA DX, file_name1

INT 21H

;reading data from file1

MOV BX, AX

MOV CX, 30

LEA DX, temp_data

MOV AH, 3FH

INT 21H

MOV temp_len, AX

;closing one file to save

MOV BX, file_hd1

MOV AH, 3EH

INT 21h

;creating another file

LEA DX, file_name2

MOV CX, 0

MOV AH, 3CH

INT 21H

MOV file_hd2, AX

;writing data to file

MOV BX, file_hd2

MOV CX, temp_len

LEA DX, temp_data

MOV AH, 40H

INT 21H

;closing file

MOV BX, file_hd2

MOV AH, 3EH

INT 21h

INT 3

code ends

end

Consider an array(array1) of 20 random numbers ranging between 1 to 4. Write a program to Count the number of 1’s, 2’s, 3’s and 4’s in the array1 and store the result of the count in one more array (array 2) of size 4 elements. At first location of array2 it has to store the count of 1’s, at second location it has to store count of 2’s and so on... 3.

data segment

array1   db  1,1,2,4,2,3,2,3,1,4,2,2,1,2,4,3,2,1,4,1

array2   db  4 dup(0)

data ends

code segment

assume cs:code, ds:data

MOV AX, data

MOV ds, AX

;counting

LEA DI, array2

ADD DI, 3

MOV BL, 4

outerloop:

LEA SI, array1

MOV BH, 0

MOV CX,20

loop1:

CMP BL, byte ptr [SI]

JNE skip

INC BH

skip:

INC SI

LOOP loop1

MOV byte ptr [DI], BH

DEC DI

DEC BL

JNZ outerloop

INT 3

code ends

end

# LAB10

1. Write an assembly language to find the LCM of two numbers. (Pass the parameters to the procedure using registers, pointers and stack)

data segment

num1    db 15

num2    db 25

anslcm  DW ?

data ends

stack1 SEGMENT STACK

DW 10 dup(0)

stacktop LABEL WORD

stack1 ends

; using direct variables

code segment

assume cs:code, ds:data, ss:stack1

MOV AX, data

MOV ds, AX

MOV AX, stack1

MOV ss, AX

MOV SP, stacktop

CALL find_lcm

INT 3

PROC find_lcm NEAR

MOV BH, num1

MOV BL, num2

MOV CX,2

next:

MOV AX, CX

DIV BH

CMP AH, 0

JNZ skip

MOV AX, CX

DIV BL

CMP AH, 0

JNZ skip

MOV anslcm, CX

RET

skip:

INC CX

CMP CX, 0FFFFH

JNZ next

find_lcm ENDP

code ends

end

; using registers

code segment

assume cs:code, ds:data, ss:stack1

MOV AX, data

MOV ds, AX

MOV AX, stack1

MOV ss, AX

MOV SP, stacktop

MOV AL, num1

MOV AH, num2

CALL find_lcm

MOV anslcm, AX

INT 3

PROC find_lcm NEAR

MOV BH, AH

MOV BL, AL

MOV CX,2

next:

MOV AX, CX

DIV BH

CMP AH, 0

JNZ skip

MOV AX, CX

DIV BL

CMP AH, 0

JNZ skip

MOV AX, CX

RET

skip:

INC CX

CMP CX, 0FFFFH

JNZ next

find_lcm ENDP

code ends

end

;using pointers

code segment

assume cs:code, ds:data, ss:stack1

MOV AX, data

MOV ds, AX

MOV AX, stack1

MOV ss, AX

MOV SP, stacktop

LEA SI, num1

LEA DI, num2

CALL find_lcm

INT 3

PROC find_lcm NEAR

MOV BH, [SI]

MOV BL, [DI]

INC DI

MOV CX,2

next:

MOV AX, CX

DIV BH

CMP AH, 0

JNZ skip

MOV AX, CX

DIV BL

CMP AH, 0

JNZ skip

MOV [DI], CX

RET

skip:

INC CX

CMP CX, 0FFFFH

JNZ next

find_lcm ENDP

code ends

end

;using stack

code segment

assume cs:code, ds:data, ss:stack1

MOV AX, data

MOV ds, AX

MOV AX, stack1

MOV ss, AX

MOV SP, stacktop

MOV AL, num1

MOV AH, num2

PUSH AX

CALL find_lcm

POP AX

MOV anslcm, AX

INT 3

PROC find_lcm NEAR

MOV BH, 2[SP]

MOV BL, 3[SP]

MOV CX,2

next:

MOV AX, CX

DIV BH

CMP AH, 0

JNZ skip

MOV AX, CX

DIV BL

CMP AH, 0

JNZ skip

MOV 2[SP], CX

RET

skip:

INC CX

CMP CX, 0FFFFH

JNZ next

find_lcm ENDP

code ends

end

2. Write an assembly language program to convert temperature from Fahrenheit to Celsius. [(F − 32) × 5/9 = °C]

data segment

tempInF dW  86

tempInC dW  ?

data ends

stack1 SEGMENT STACK

DW 10 dup(0)

stacktop LABEL WORD

stack1 ends

code segment

assume cs:code, ds:data, ss:stack1

MOV AX, data

MOV ds, AX

MOV AX, stack1

MOV ss, AX

MOV SP, stacktop

CALL convertToF

INT 3

PROC convertToF NEAR

MOV AX, tempInF

MOV BX, 32

SUB AX, BX

MOV BX, 5

MUL BX

MOV BX, 9

DIV BL

MOV tempInC, AX

RET

convertToF ENDP

code ends

end

# LAB11

1. Write an assembly language program using far procedure to find square root of a perfect square number using procedure.

; procedure for square root

public findsqrt

sqrtt segment

assume cs:sqrtt

findsqrt PROC far

; BX <- number to find square

MOV CX, 0

sqr_loop:

INC CL

CMP CL, AL

JNC end1

MOV AX, BX

DIV CL

CMP AH, 0

JNE sqr_loop

CMP AL, CL

JNE sqr_loop

MOV AX, CX

end1:

RET

;AX <- answer

findsqrt ENDP

sqrtt ends

end

extrn findsqrt:far

data segment

num1    dw  16

num2    dw  25

ans1    db  ?

ans2    db  ?

data ends

code segment

assume cs: code,ds:data

mov ax,data

mov ds,ax

mov bx, num1

call findsqrt

mov ans1, AL

int 03h

code ends

end

2. Write a program using Macro to add two numbers. Macro should be written in another file.

SUM MACRO a, b

mov ax, a

mov bx, b

add ax, bx

ENDM

INCLUDE ex22.h

data segment

num1    dw  6

num2    dw  9

ans dw ?

data ends

code segment

assume cs:code, ds:data

mov ax, data

mov ds,  ax

SUM num1, num2

mov ans, ax

INT 03

code ends

end

3. Write an assembly language to override the overflow interrupt.

data segment

msg db "Overflow Occured",13,10,"$"

data ends

stacks segment

stack dw 40 dup(0)

top label word

stacks ends

code segment

assume cs:code,ds:data,ss:stacks

mov ax,data

mov ds,ax

mov ax,stacks

mov ss,ax

mov sp,top

mov ax,0000

mov es,ax

mov word ptr es:0010h,offset overflow

mov word ptr es:0012h,seg overflow

mov al,100

add al,100

INTO

int 03h

overflow proc near

lea dx, msg

mov ah, 9

int 21h

iret

overflow endp

code ends

end